Minnesota State University, Mankato

Electrical and Computer Engineering Department

Lab Assignment 5 – Fourier Transform of an Energy Signal

Date of experiment: 02/18/2025

Date of Submission: 02/25/2025

Student name

Nathnael Minuta

TA name

Masamune Oso

EE 363-01 Communication Systems Laboratory

Dr. Yikai Li

Spring 2025

Contents

Pages

Abstract ……………………………………………………………………………. 3

Materials…………………………………………..………………………………  3

Procedure, Data Collection, and Analysis……….…………………   3

Conclusion…………………………………………..……………………………   11

Appendices…………………………………………..……………………………  12

References…………………………………………..……………………………   18

Fourier Transform of an Energy Signal

Abstract

This lab investigates the Fourier Transform of a Gaussian-shaped energy signal using numerical integration, avoiding built-in Fast Fourier Transform (FFT) functions. The objective is to analyze how a time-domain energy signal behaves in the frequency domain. The experiment includes computing the magnitude and phase spectra, along with the Energy Spectrum Density (ESD). Additionally, we examine how time-shifting x(t−3T) and time-scaling x(2t) affect the Fourier Transform. The results validate the fundamental properties of the Fourier Transform, such as time-frequency duality and shift invariance.

Materials

Equipment Needed                                        Quantity.

Laptop (Visual Studio Code)                                  1

Methods (Procedure, Data Collection, and Analysis)

Part A: Computing the Fourier Transform of an Energy Signal

- Define an energy-limited signal:

where T=0.1ms and t ranges from 0 to 10T.

- Implement numerical integration to compute its Fourier Transform:

- Compute and plot:

  - Magnitude Spectrum ∣X(f)∣

  - Phase Spectrum ∠X(f)

  - Energy Spectrum Density ∣X(f)∣2

Part B: Fourier Transform of Time-Shifted Signal x(t−3T)

- Compute the signal shifted by 3T:

- Compute and plot:

  - Magnitude Spectrum ∣Xshifted(f)∣

  - Phase Spectrum ∠Xshifted(f)

- Compare the phase shift with the original spectrum.

Part C: Fourier Transform of Scaled Signal x(2t)

- Compute the time-scaled signal:

- Compute and plot:

  - Magnitude Spectrum ∣Xscaled(f)∣

  - Phase Spectrum ∠Xscaled(f)

- Compare the spectral compression/stretching with the original spectrum.

Results

Conclusion

In this lab, we successfully implemented the Fourier Transform of an energy signal using numerical integration and analyzed its behavior in the frequency domain.

Our key observations include:

- Magnitude spectra remain unchanged under time shifts, but phase shifts occur as expected.

- Time scaling compresses the signal in time while stretching it in the frequency domain, confirming time-frequency duality.

- The computed Energy Spectrum Density ∣X(f)∣2 shows how the signal distributes its energy over frequencies.

Challenges in this lab included handling numerical integration accuracy and ensuring a sufficiently large frequency range to capture the signal’s spectral characteristics. Overall, this experiment reinforced core Fourier Transform concepts and their applications in signal processing.

Appendices

References

- https://rodzah.files.wordpress.com/2011/07/how-to-write-lab-report.pdf

- EE343- LAB Assignment 5.pdf